Advanced Project in Robotics 2026: Waste-Sorter

Jan Sistonen & Jermu Roivanen

Contents

Introduction and conceptual plan	2

System Overview	3

Hardware Components	3

Waste Classification Options	3

Technical implementation of the project	4

Machine Learning and Dataset	5

Implementation Phases	5

Hardware Components	6

Software and Environment	6

Integration and Interfaces	7

Mechanical Design and Software Logic	7

Future Improvements	7

Links	8

Contribution and evaluation	8

Hour management	9

Evaluation	9

References	9

Introduction and conceptual plan

We wanted to choose an interesting, yet meaningful topic for our project. After considering several options, we selected a challenging and necessary topic for our project: waste sorting. Scientific studies show that people often put waste in the incorrect bins in public areas. Human error decreases the quality of recycled materials.  In a recent field study conducted at four public locations in two Austrian cities, only approximately 20% of recyclable materials were disposed of in the correct waste fraction (Kladnik, V. et al., 2025). Other studies conducted at university campuses and public events have reported waste-stream contamination rates of approximately 23–44% (Hottle, T. et al., 2015). Incorrect sorting reduces the quality and value of separately collected materials, increases processing costs, and may result in contaminated bins or loads being treated as residual waste instead of being recycled.

Because of this, it is on demand to sort waste automatically at the source, not only at the waste management facility. Our robot-assisted waste sorting solution would resolve this issue by handling the sorting process on behalf of the users.

The largest challenge in this project is to identify the different types of waste correctly. Other challenges include the mechanical design and the integration of hardware and software.

The conceptual plan details the development of a Waste Sorting Robot for an advanced robotics project.

## System Overview

The primary goal of the project is to build a system that processes input consisting of mixed waste items placed on a garbage bin. The system uses a modular architecture. It has three main parts: perception, control, and actuation.

First, a vision system evaluates and classifies the specific waste type. Following classification, the system's control logic routes the item to the correct sorting bin.

## Hardware Components

The physical operation of the sorting robot is built upon several key hardware components:

- A Raspberry Pi serves as the central system controller.

- A USB camera is utilized for image capture.

- The items are transported using a motorized bin by using a stepper motor and timing belt. Servo is used for basket opening.

- Object detection is managed by a USB camera, and bin detection is managed by using an ultrasonic sensor.

heavily on testing the perception modules.

## Waste Classification Options

To handle the image analysis, the project explores five different waste classification options. These include utilizing the cloud-based Google Vision API, implementing pre-trained or custom-trained models, or building a custom machine learning model trained on a specific project dataset. The final decision will be based on a thorough comparison of each option's accuracy, latency, and complexity.

# Technical implementation of the project

The system follows a Sense-Think-Act pipeline.

- When you start the program, the system always does a "homing" sequence. The stepper motor moves the basket back until the ultrasonic sensor detects it. This puts the basket in the exact starting position under the camera.

- The user puts an item into the basket.

- The USB webcam captures the image. The YOLO model identifies the item and the material class.

- The Raspberry Pi tells the stepper motor to move the basket to the correct waste bin.

- The servo motor opens the hatch to drop the item.

- The servo motor closes the hatch, and the basket returns to the home position.

## Machine Learning and Dataset

To identify the waste, we built a custom machine learning model. We used a pre-trained YOLO model (best.pt) and the OpenCV library.

We collected our own images and used Roboflow to annotate the custom dataset. We created a specific naming rule for the labels. The labels use a prefix to identify the waste material class. For example, we labeled a banana as bio_banana. The Python software reads the prefix bio and knows that the object is organic waste. This logic tells the system which bin to select.

## Implementation Phases

We built the project in five primary phases:

Phase 1: Initial Software Prototype First, we made the dataset and trained the model. We tested the object detection to see if the YOLO model and OpenCV operated correctly. We did these initial tests on a personal laptop with the Ubuntu 22.04 operating system.

Phase 2: Basic Hardware Control Next, we started using Raspberry Pi. We learned to control simple components like LEDs and the servo motor. We learned how to connect to the hardware and how to use the gpiozero Python library.

Phase 3: Stepper Motor Integration We installed the NEMA 17 stepper motor and its motor driver. We studied the motor documentation. We used a scientific formula to calculate and adjust the correct electrical current limit for the motor driver.

Phase 4: System Integration We downloaded our custom YOLO model to the Raspberry Pi. We wrote the Python code to connect the camera, the machine learning model, and the Raspberry Pi. The Raspberry Pi had sufficient processing power to run the model smoothly. We connected the motors and the control logic. The full system operated correctly.

Phase 5: Physical Assembly Finally, we built the physical frame from 4040 aluminum profiles. We used CAD software to make 3D models for the custom parts. We 3D-printed these parts ourselves and attached all components to the frame.

## Hardware Components

The physical sorting mechanism and control system rely on the following hardware specifications:

- Controller: A Raspberry Pi 5 functions as the central processing unit.

- Vision: A USB Webcam is used to capture visual data.

- Proximity and Positioning: An ultrasonic sensor is responsible for presence detection and distance measurement during basket movement.

- Actuators: A NEMA 17 stepper motor moves the sorting basket linearly on a timing belt. A servo motor opens and closes the hatch on the bottom of the basket.

- Power: The system is powered by a 5V/5A power supply for the Raspberry Pi and requires a battery for stepper motor and Servo. For those we are using 3 cell, 11.1V, 1200 mAh, 20C high discharge LI-PO battery.

## Software and Environment

- Operating System: The system runs on a 64-bit Raspberry Pi OS.

- Programming and Communication: Python is used to program the machine learning logic and manage sensor inputs.

- Machine Learning Model: The vision system will rely on a pre-trained model YOLO, fine-tuned on a targeted waste dataset containing plastic, bio, metal, cartboard, and mixed wasted.

- Operating Environment: The sorter is designed for indoor use and requires consistent lighting, as shifting shadows can disrupt the machine learning model's accuracy. Development and system management are handled remotely via an SSH connection to the Pi.

## Integration and Interfaces

The hardware components communicate with the central controller through specific interfaces:

- Camera: Connects to the Pi via a USB 3.0 or MIPI CSI interface.

- Ultrasonic Sensor: Interfaces with the Pi using GPIO pins, specifically utilizing Trigger and Echo signals.

- Actuators: The NEMA 17 stepper motor was controlled using an A4988 motor driver connected to the Raspberry Pi GPIO pins.

The driver receives STEP and DIR signals from the Raspberry Pi and regulates the current supplied to the motor windings. Before operation, the A4988 current limit was calibrated according to the manufacturer’s datasheet. The required reference voltage was calculated using

VREF=8RSITripMAX

Where (R_S) is the current-sense resistance of the driver module and ITripMAX is the desired maximum motor phase current. The reference voltage was measured between the driver’s VREF test point and ground using a multimeter, and the onboard potentiometer was adjusted until the calculated value was reached.

## Mechanical Design and Software Logic

The software logic operated very well from the beginning. However, the mechanical parts required many design iterations.

It was difficult to move the physical sorting basket smoothly on the rails. We built many different mechanical models to change the rotary movement of the stepper motor into linear movement with the timing belt. We tested and discarded many 3D-printed parts before we found a stable solution.

[laitan tähän kuvan ekasta hihnaversiosta]

[kuva eri ‘turhista’ 3d tulostuksista / iteraatioista]

## Future Improvements

In the future, we can improve how the system classifies waste. For this project, we used our custom dataset. We also tested ready-made public datasets (for example, the TACo dataset). The ready-made datasets identified individual objects very accurately. However, they did not contain data about the material quality or the waste class. Also, we wanted to see if we managed to implement the full system from scratch.

A future solution is to combine a standard object detection model with a Large Language Model (LLM).

- The standard model identifies the object (for example, "banana").

- The system sends the word "banana" to the LLM.

- The system asks the LLM: "What waste class is this?"

- The LLM answers "organic waste".

- The system uses this answer to move the motor to the correct bin.

This solution removes the need to annotate thousands of images manually for waste material classes.

## Links

Here are links to our robot introduction video and project Github.

Video: waste-sort-final-edit.mp4

Github: https://github.com/jansistonen/waste-sorter

# Contribution and evaluation

### Hour management

| Member | Hours | Topic |
| --- | --- | --- |

### Evaluation

We managed to meet the minimum expectations and goals for our project. The system and control logic work flawlessly. We’d still need to apply safety logic and prevent possible race conditions. Physical testing was quite limited. Software threading needs to be applied, so that the camera can keep delivering video feed even when the motor functions are called (would not be necessary in a “real product” without a screen, but possible for remote debugging).

Second, the goal was to “ROSsify” the system, so that it would be more industrially ready. That we did not yet do, but it would be interesting to translate the “one-single-Python-script" to working ROS nodes, even without the physical environment.

We self-evaluate our project to grade 5. The final solution is not flawless, but we wanted to challenge ourselves and learn as much as possible. When doing a lot of stuff from scratch, it took its own time, but for that reason we got a clear whole picture how our robot works. We are quite happy how we manage to build functioning waste sorter, and we have a lot of improving ideas.

# References

Kladnik, V., Schwarzböck, T., & Dworak, S. (2025). Improving waste separation in public spaces – A field study from Austrian cities. Detritus, 31, 3–16. doi:10.31025/2611-4135/2025.19490.

Hottle, T. A., Bilec, M. M., Brown, N. R., & Landis, A. E. (2015). Toward zero waste: Composting and recycling for sustainable venue based events. Waste Management, 38, 86–94. doi:10.1016/j.wasman.2015.01.019.